Supplementary Materials for the article:

Extra-matrix Mg2+ limits Ca2+ uptake and modulates Ca2+ uptake–independent respiration and redox state in cardiac isolated mitochondria

Age D. Boelens1, Ranjan K. Pradhan2,3, Christoph A. Blomeyer1,

Amadou K. S. Camara1,4, Ranjan K. Dash2,3, David F. Stowe1,2,4-6

1Department of Anesthesiology, 2Department of Physiology, 3Biotechnology and Bioengineering Center and 4Cardiovascular Research Center, Medical College of Wisconsin, Milwaukee, Wisconsin 53226, USA; 5Research Service, Zablocki Veterans Affairs Medical Center, Milwaukee, Wisconsin, 53295, USA; 6Department of Biomedical Engineering, Marquette University, Milwaukee, Wisconsin, 53201, USA

Determination of Ca2+ Kd for indo-1

The Kd for indo-1 on its introduction as a marker for intracellular [Ca2+] was reported to be approximately 250 nM [Ca2+] at pH 7.05 at 37°C. It is well known that temperature, protein concentration, and pH can greatly influence the apparent Kd of indo-1 (Grynkiewicz et al 1985). Therefore, these factors must be measured and accounted for in determining the Kd for a particular set of experiments. We determined the Kd for indo-1 AM and indo-1 pentapotassium (PP) salt dye under our specific experimental conditions based on a modified protocol by Petr et al. (Petr et al 1997). We used a calibration kit provided by Invitrogen (C3008MP), which contained two bottles of 50 ml calibration solution with 100 mM KCl and 30 mM MOPS at pH 7.2, which approximate our experimental buffer conditions at 25 °C. We used 10 mM K2-EGTA (0 [Ca2+] and 10 mM Ca-EGTA (maximal [Ca2+] of 39 µM) to determine min and max values of Ca2+. These stock solutions were mixed to produce solutions within this range. Free [Ca2+] was calculated using the formula:

Free [Ca2+] = KdEGTA • [Ca-EGTA]/[K2-EGTA]

Mitochondria were loaded with either indo-1-AM or DMSO (for background measurement) at 0.5 mg protein/ml, and were suspended in calibration solution with 11 different free [Ca2+]. Ionomycin and CCCP were present to equilibrate Ca2+ and protons (pH), respectively, across the intra-mitochon-drial membrane (IMM). Continuous emission (λem) scans were recorded over the range 380 to 500 nm at excitation (λex) 350 nm. Representative traces of emission scans from 0 to 10 mM Ca-EGTA are illustrated in Fig. S1.

Fig. S1. Fluorescence intensities (F) were measured at 456 nm and 390 nm and R was calculated by dividing F390/F456. 0 µM free Ca2+ was used to determine the ratio when all of the dye was unbound (Rmin) and 39 µM free Ca2+ for ratio when all of the dye was bound (Rmax). In a plot of –log [Ca2+] on the x-axis vs. –log β[(R-Rmin)/(Rmax-R) on the y-axis, the data points formed a straight line with the x-intercept representing the pKd (Fig. S2). The Kd for indo-1-AM inside the mitochondrial matrix (m) was determined as 326 ± 20 nM [Ca2+]m with n =5.

We similarly determined the Kd for indo-1-PP salt in the presence of mitochondria under the same buffer conditions (figure not shown). For these experiments indo-1-PP salt was added to the calibration solutions directly. Mitochondria, thus not loaded with indo-1-AM, were added at 0.5 mg protein/ml. No ionomycin or CCCP was added because Ca2+ and protons did not need to be equilibrated. Emission scans were performed and data was processed as for indo-1-AM. The Kd for indo-1-PP in the external (e) medium was determined as 311 ± 24 nM [Ca2+]e with n =3.

Fig. S2. Determination of pKd for indo 1-AM

Added MgCl2 reduces state 3 respiration via adenine nucleotide translocase

ATP/ADP (adenyl nucleotide) translocase (ANT) enables ATP4- and ADP3- to be exchanged across the mitochondrial inner membrane and requires consumption of one mole of H+ for each mole of nucleotide exchanged. Atractyloside (ATR) binds to ANT locking the cytosolic side in the open confirmation. Fig. S3 shows stepwise inhibition of state 3 respiration by ATR in the presence and absence of 2 mM MgCl2 with 0.6 mM CaCl2. Note that at 1 µM ATR, 2 mM MgCl2 reduced O2 consumption by about 20%; 1 mM MgCl2 with 0.6 mM CaCl2 reduced O2 consumption by about 10% (Fig. 9A). This effect to reduce state 3 respiration may be due in part to greater binding of Mg2+ to nucleotides, which impedes ADP/ATP translocation.

Added MgCl2 enhances state 4 respiration via Mg2+-dependent ATPase

In isolated mitochondrial experiments state 4 is generally faster than state 2 because of residual ATP-ases that convert the newly formed ATP to ADP, which stimulates respiration. A slowing of state 4 respiration by oligomycin (OMN) may indicate that hydrolysis of newly formed ATP accounts for some of this effect. Moreover, since OMN blocked the effect of added MgCl2 to enhance state 4 respiration (Fig. S4), there may be a role for higher [Mg2+] to enhance Mg2+ dependent ATPase activity.

Fig. S4. Effect of oligomycin (OMN) to reverse increase in state 4 respiration by 1 mM MgCl2.

Effect of Mg2+ to reduce state 3 respiration when Ca2+ uptake is blocked by ruthenium red

To further demonstrate that the inhibiting effect of MgCl2 on Ca2+-stimulated state 3 respiration is mediated in part by elevated matrix [Ca2+], we conducted additional experiments in which MgCl2 was added at different buffer [CaCl2 ], but this time in the presence of 1 µM ruthenium red (RR) to completely block all mCa2+ uptake by the Ca2+ uniporter (CU) (Cox and Matlib, 1993; McCormack et al, 1989). Our original protocol (Fig. 1) was amended to add RR to the respiration buffer before adding CaCl2 or MgCl2 (Fig. S5) to prevent any Ca2+uptake. For these supplemental experiments, [MgCl2] were 0, 0.125, 1 and 2 mM and [CaCl2] were 0, 0.4 and 0.6 mM. For each [CaCl2], RR completely inhibited the [Ca2+]m-induced increase in state 3 respiration (Fig. S6) we observed in the absence of RR. Moreover, this effect of RR on state 3 respiration was not only observed in the absence, but also in the presence of each [MgCl2] so that there was no difference among the groups (Fig. S6). This supplemental data agrees with the no added Ca2+ data shown in Fig. 9A,C). That is, under conditions where there was no Ca2+ uptake adding MgCl2 did not alter state 3 respiration.

Fig. S6. State 3 respiration (n=3 hearts) at different [MgCl2] and [CaCl2] after adding 1 µM ruthenium red.

Ruthenium Red does not alter the external Mg2+-induced increase in state 4 respiration

In the presence of RR at any [CaCl2], RR did not block the increase in state 4 respiration observed by adding MgCl2 (Fig. S7). This further confirmed that this indirect effect of Mg2+ to enhance state 4 respiration is independent of any change in [Ca2+]m. Overall, these supplemental findings with RR agree with our results shown in Fig. 9B,D, i.e., increasing [MgCl2] enhanced state 4 respiration both in the absence (Fig. 9B,D) and in the presence (Fig. S7) of RR. This effect of Mg2+ on state 4 respiration independent of any change in [Ca2+]m supports our contention that the increase in state 4 respiration by MgCl2 was caused by stimulation of extra-matrix ATPases (Fig. S4).

Fig. S7. State 4 respiration (n=3 hearts) at different [MgCl2] and [CaCl2] after adding 1 µM ruthenium red.

References

Cox DA, Matlib MA (1993) A role for the mitochondrial Na+-Ca2+ exchanger in the regulation of oxidative phosphorylation in isolated heart mitochondria. J Biol Chem 268(2);938-947

Grynkiewicz G, Poenie M, Tsien RY (1985) A new generation of Ca2+ indicators with greatly improved fluorescence properties. J Biol Chem 260(6);3440-3450

McCormack JG, Browne HM, Dawes NJ (1989) Studies on mitochondrial Ca2+-transport and matrix Ca2+ using fura-2-loaded rat heart mitochondria. Biochim Biophys Acta 973(3);420-427

Petr MJ, Wurster RD (1997) Determination of in situ dissociation constant for Fura-2 and quantitation of background fluorescence in astrocyte cell line U373-MG. Cell Calcium 21(3);233-240